How to Make RJ45 crossover adaptors

Pinouts

Matrix X-Over	(with the TX/RX data lines looped together)

| Connector 1 | Connector 2 |
| --- | --- |
| Pin 1 (Matrix RX Data +ve) | Pin 7 |
| Pin 2 (Matrix RX Data -ve) | Pin 8 |
| Pin 3 (Matrix Audio RX +ve) | Pin 4 |
| Pin 4(Matrix Audio TX +ve) | Pin 3 |
| Pin 5(Matrix Audio TX -ve) | Pin 6 |
| Pin 6 (Matrix Audio RX -ve) | Pin 5 |
| Pin 7 (Matrix TX Data +ve) | Pin 1 |
| Pin 8 (Matrix TX Data -ve) | Pin 2 |
| Screen | Screen |

Audio X-Over	(without the TX/RX data lines looped together)

| Connector 1 | Connector 2 |
| --- | --- |
| Pin 1 (Matrix RX Data +ve) | No Connection |
| Pin 2 (Matrix RX Data -ve) | No Connection |
| Pin 3 (Matrix Audio RX +ve) | Pin 4 |
| Pin 4(Matrix Audio TX +ve) | Pin 3 |
| Pin 5(Matrix Audio TX -ve) | Pin 6 |
| Pin 6 (Matrix Audio RX -ve) | Pin 5 |
| Pin 7 (Matrix TX Data +ve) | No Connection |
| Pin 8 (Matrix TX Data -ve) | No Connection |
| Screen | Screen |

E1 X-Over	(when link EQUE card E1/T1 back to back or some 3rd party devices)

| Connector 1 | Connector 2 |
| --- | --- |
| Pin 1 (Data TX +ve) | Pin 4 |
| Pin 2(Data TX -ve) | Pin 5 |
| Pin 3 | No Connection |
| Pin 4(Data RX +ve) | Pin 1 |
| Pin 5(Data RX -ve) | Pin 2 |
| Pin 6 | No Connection |
| Pin 7 | No Connection |
| Pin 8 | No Connection |
| Screen | Screen |

Construction

The construction process is identical for all three types:

- Take two PCB Mount RJ45 connectors

- Carefully align back to back and clamp in a vice

- Solder the shields together on all four sides

- Trim off the PCB mounting lugs and wire the connectors according to the appropriate table

- After wiring trim the legs

6. Cover in Heat Shrink, trim to a neat finish and apply the appropriate label